DESIGN CODE

module smart_parking (

input clk,           // 100 MHz System Clock (Pin F14)

input reset,         // Reset Button (Pin J2)

input entry_btn,     // Entry Button (Pin J5)

input exit_btn,      // Exit Button  (Pin H2)

output reg full_led, // Discrete LED (Pin G1)

output reg [6:0] seg,// 7-Segment Cathodes (Pins B5-D7)

output wire [7:0] an // Digit Enable Anodes (Pins E4-D5)

);

parameter TOTAL_SLOTS = 8;

reg [3:0] car_count;

wire [3:0] available_slots;

// Available slots calculation

assign available_slots = TOTAL_SLOTS - car_count;

// Enable the rightmost 7-segment digit (Active-LOW)

assign an = 8'b1111_1110;

// -------------------------------------------------------------------------

// 1. DEBOUNCE SAMPLING TICK GENERATOR (10 ms at 100 MHz)

// -------------------------------------------------------------------------

// 100,000,000 Hz / 1,000,000 = 100 Hz (10 ms period)

reg [19:0] debounce_counter = 20'd0;

reg sample_tick = 1'b0;

always @(posedge clk) begin

if (reset) begin

debounce_counter <= 20'd0;

sample_tick      <= 1'b0;

end else if (debounce_counter >= 20'd999_999) begin

debounce_counter <= 20'd0;

sample_tick      <= 1'b1;

end else begin

debounce_counter <= debounce_counter + 1'b1;

sample_tick      <= 1'b0;

end

end

// -------------------------------------------------------------------------

// 2. 2-STAGE METASTABILITY SYNCHRONIZERS

// -------------------------------------------------------------------------

reg entry_sync_0, entry_sync_1;

reg exit_sync_0,  exit_sync_1;

always @(posedge clk) begin

if (reset) begin

entry_sync_0 <= 1'b0;

entry_sync_1 <= 1'b0;

exit_sync_0  <= 1'b0;

exit_sync_1  <= 1'b0;

end else begin

entry_sync_0 <= entry_btn;

entry_sync_1 <= entry_sync_0;

exit_sync_0  <= exit_btn;

exit_sync_1  <= exit_sync_0;

end

end

// -------------------------------------------------------------------------

// 3. DEBOUNCE FILTER & EDGE DETECTORS

// -------------------------------------------------------------------------

reg entry_debounced, exit_debounced;

reg entry_last,      exit_last;

always @(posedge clk) begin

if (reset) begin

entry_debounced <= 1'b0;

exit_debounced  <= 1'b0;

entry_last      <= 1'b0;

exit_last       <= 1'b0;

end else begin

// Sample synchronized button inputs every 10 ms

if (sample_tick) begin

entry_debounced <= entry_sync_1;

exit_debounced  <= exit_sync_1;

end

// Delay register to create a 1-clock pulse on rising edge

entry_last <= entry_debounced;

exit_last  <= exit_debounced;

end

end

// Single-cycle active-high pulses

wire entry_pulse = (entry_debounced && !entry_last);

wire exit_pulse  = (exit_debounced  && !exit_last);

// -------------------------------------------------------------------------

// 4. CAR COUNTER LOGIC (ENTRY & EXIT)

// -------------------------------------------------------------------------

always @(posedge clk) begin

if (reset) begin

car_count <= 4'd0;

end else begin

// Car Entry: Increment car count (decrease open spots) up to TOTAL_SLOTS

if (entry_pulse && !exit_pulse && (car_count < TOTAL_SLOTS)) begin

car_count <= car_count + 1'b1;

end

// Car Exit: Decrement car count (increase open spots) down to 0

else if (exit_pulse && !entry_pulse && (car_count > 0)) begin

car_count <= car_count - 1'b1;

end

end

end

// -------------------------------------------------------------------------

// 5. FULL LOT INDICATOR LED (Active-HIGH)

// -------------------------------------------------------------------------

always @(posedge clk) begin

if (reset) begin

full_led <= 1'b0;

end else if (car_count == TOTAL_SLOTS) begin

full_led <= 1'b1;

end else begin

full_led <= 1'b0;

end

end

// -------------------------------------------------------------------------

// 6. 7-SEGMENT DECODER (Displays Available Slots, Active-LOW)

// -------------------------------------------------------------------------

always @(*) begin

case (available_slots)

4'd0: seg = 7'b1000000; // 0 slots left (Full)

4'd1: seg = 7'b1111001; // 1 slot left

4'd2: seg = 7'b0100100; // 2 slots left

4'd3: seg = 7'b0110000; // 3 slots left

4'd4: seg = 7'b0011001; // 4 slots left

4'd5: seg = 7'b0010010; // 5 slots left

4'd6: seg = 7'b0000010; // 6 slots left

4'd7: seg = 7'b1111000; // 7 slots left

4'd8: seg = 7'b0000000; // 8 slots left (Empty)

default: seg = 7'b1111111; // All OFF

endcase

end

endmodule

TESTBENCH

`timescale 1ns/1ps

module tb_smart_parking;

reg clk;

reg reset;

reg entry_btn;

reg exit_btn;

wire full_led;

wire [6:0] seg;

wire [7:0] an;

smart_parking DUT (

.clk(clk),

.reset(reset),

.entry_btn(entry_btn),

.exit_btn(exit_btn),

.full_led(full_led),

.seg(seg),

.an(an)

);

initial clk = 0;

always #5 clk = ~clk;

initial begin

reset = 1;

entry_btn = 0;

exit_btn  = 0;

#20 reset = 0;

repeat (8) begin

#10 entry_btn = 1;

#10 entry_btn = 0;

end

#50;

$finish;

end

initial begin

$monitor("T=%0t | cars=%0d | FULL_LED=%b",

$time, DUT.car_count, full_led);

end

initial begin

$dumpfile("tb_smart_parking.vcd");

$dumpvars;

end

endmodule

CONSTRAINTS

## --------------------------------------------------------------------------

## Clock Signal (100MHz)

## --------------------------------------------------------------------------

set_property PACKAGE_PIN F14 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

## --------------------------------------------------------------------------

## Buttons (Pushbuttons)

## --------------------------------------------------------------------------

# Reset (Button 0)

set_property PACKAGE_PIN J2 [get_ports reset]

set_property IOSTANDARD LVCMOS33 [get_ports reset]

# Entry (Button 1)

set_property PACKAGE_PIN J5 [get_ports entry_btn]

set_property IOSTANDARD LVCMOS33 [get_ports entry_btn]

# Exit (Button 2)

set_property PACKAGE_PIN H2 [get_ports exit_btn]

set_property IOSTANDARD LVCMOS33 [get_ports exit_btn]

## --------------------------------------------------------------------------

## Discrete LED (Full Indicator)

## --------------------------------------------------------------------------

set_property PACKAGE_PIN G1 [get_ports full_led]

set_property IOSTANDARD LVCMOS33 [get_ports full_led]

## --------------------------------------------------------------------------

## 7-Segment Cathodes (seg[0] to seg[6])

## Mapping according to Figure 6: seg[0]=a (D0_seg0), ..., seg[6]=g (D0_seg6)

## --------------------------------------------------------------------------

set_property PACKAGE_PIN D7 [get_ports {seg[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}]

set_property PACKAGE_PIN C5 [get_ports {seg[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}]

set_property PACKAGE_PIN A5 [get_ports {seg[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}]

set_property PACKAGE_PIN B7 [get_ports {seg[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}]

set_property PACKAGE_PIN A7 [get_ports {seg[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}]

set_property PACKAGE_PIN D6 [get_ports {seg[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}]

set_property PACKAGE_PIN B5 [get_ports {seg[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}]

## --------------------------------------------------------------------------

## Clock Signal (100MHz)

## --------------------------------------------------------------------------

set_property PACKAGE_PIN F14 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

## --------------------------------------------------------------------------

## Buttons (Pushbuttons)

## --------------------------------------------------------------------------

# Reset (Button 0)

set_property PACKAGE_PIN J2 [get_ports reset]

set_property IOSTANDARD LVCMOS33 [get_ports reset]

# Entry (Button 1)

set_property PACKAGE_PIN J5 [get_ports entry_btn]

set_property IOSTANDARD LVCMOS33 [get_ports entry_btn]

# Exit (Button 2)

set_property PACKAGE_PIN H2 [get_ports exit_btn]

set_property IOSTANDARD LVCMOS33 [get_ports exit_btn]

## --------------------------------------------------------------------------

## Discrete LED (Full Indicator)

## --------------------------------------------------------------------------

set_property PACKAGE_PIN G1 [get_ports full_led]

set_property IOSTANDARD LVCMOS33 [get_ports full_led]

## --------------------------------------------------------------------------

## 7-Segment Cathodes (seg[0] to seg[6])

## Mapping according to Figure 6: seg[0]=a (D0_seg0), ..., seg[6]=g (D0_seg6)

## --------------------------------------------------------------------------

set_property PACKAGE_PIN D7 [get_ports {seg[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}]

set_property PACKAGE_PIN C5 [get_ports {seg[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}]

set_property PACKAGE_PIN A5 [get_ports {seg[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}]

set_property PACKAGE_PIN B7 [get_ports {seg[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}]

set_property PACKAGE_PIN A7 [get_ports {seg[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}]

set_property PACKAGE_PIN D6 [get_ports {seg[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}]

set_property PACKAGE_PIN B5 [get_ports {seg[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}]

## --------------------------------------------------------------------------

## 7-Segment Anodes (an[0] to an[7])

## All 8 ports defined to fix [DRC NSTD-1] and [DRC UCIO-1] errors

## --------------------------------------------------------------------------

# Rightmost Display (D0_a0 to D0_a3)

set_property PACKAGE_PIN D5 [get_ports {an[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]

set_property PACKAGE_PIN C4 [get_ports {an[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

set_property PACKAGE_PIN C7 [get_ports {an[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

set_property PACKAGE_PIN A8 [get_ports {an[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

# Leftmost Display (D1_a0 to D1_a3)

set_property PACKAGE_PIN H3 [get_ports {an[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[4]}]

set_property PACKAGE_PIN J4 [get_ports {an[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[5]}]

set_property PACKAGE_PIN F3 [get_ports {an[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[6]}]

set_property PACKAGE_PIN E4 [get_ports {an[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[7]}]

## --------------------------------------------------------------------------

## 7-Segment Anodes (an[0] to an[7])

## All 8 ports defined to fix [DRC NSTD-1] and [DRC UCIO-1] errors

## --------------------------------------------------------------------------

# Rightmost Display (D0_a0 to D0_a3)

set_property PACKAGE_PIN D5 [get_ports {an[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]

set_property PACKAGE_PIN C4 [get_ports {an[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

set_property PACKAGE_PIN C7 [get_ports {an[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

set_property PACKAGE_PIN A8 [get_ports {an[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

# Leftmost Display (D1_a0 to D1_a3)

set_property PACKAGE_PIN H3 [get_ports {an[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[4]}]

set_property PACKAGE_PIN J4 [get_ports {an[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[5]}]

set_property PACKAGE_PIN F3 [get_ports {an[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[6]}]

set_property PACKAGE_PIN E4 [get_ports {an[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[7]}]